FPGA Neural Network Letter Detector

Complete Implementation Guide

Arty Z7-20 Board  |  Vitis HLS  |  Vivado  |  Vitis IDE

Covers: Training  |  HLS Synthesis  |  Block Design  |  Bare-Metal C  |  Python UI

# Project Overview

This guide covers the complete implementation of an FPGA-accelerated neural network that detects a specific letter (D, F, Y, or any EMNIST letter) drawn on a Python canvas. The system uses:

- PyTorch for training the binary classifier on EMNIST letters dataset

- Vitis HLS to synthesize the neural network as hardware IP

- Vivado to build the block design and generate the bitstream for Arty Z7-20

- Vitis IDE to write bare-metal C code that runs on the ARM Cortex-A9

- Python canvas UI to draw letters and receive results over UART

## System Architecture

| Component | Description | Technology |
| --- | --- | --- |
| Python PC | Draw letter, send 784 bytes over UART | canvas_ui.py + pyserial |
| PS (ARM) | Receive UART, copy to DDR, trigger HLS IP | Bare-metal C on Cortex-A9 |
| PL (FPGA) | Run neural network inference | HLS IP: 784->64->32->1 |
| AXI GPIO | Drive LD0/LD1 LEDs with result | Vivado AXI GPIO IP |
| UART | 784 bytes PC->Board, 1 byte result back | 115200 baud, COM port |

## EMNIST Letter Labels

Change D_LABEL in train.py, image.py, and verify_fixed.py to detect a different letter:

| Label Number | Letter |
| --- | --- |
| 4 | D |
| 6 | F |
| 8 | H |
| 25 | Y |
| 1-26 | A through Z (1=A, 2=B, ... 26=Z) |

STAGE 1: Python Training

# Stage 1 — Train the Neural Network

## Step 1.1 — Configure the Target Letter

Open train.py and change D_LABEL to the EMNIST label of the letter you want to detect:

D_LABEL = 4   # 4=D, 6=F, 25=Y — change this for different letters

Also change the same value in image.py and verify_fixed.py.

## Step 1.2 — Run Training

python train.py

This trains for 10 epochs on the EMNIST letters dataset. The network architecture is:

- Input: 784 pixels (28x28 image flattened)

- Hidden Layer 1: 784 -> 64 neurons, ReLU activation

- Hidden Layer 2: 64 -> 32 neurons, ReLU activation

- Output: 32 -> 1 neuron, Sigmoid activation

- Loss: Binary Cross Entropy

- Optimizer: Adam, LR=0.001

Output saved to model.pth. Expected final accuracy: above 95%.

## Step 1.3 — Export Fixed-Point Weights

python export_weights.py

Converts PyTorch float32 weights to Q6.10 fixed-point format (ap_fixed<16,6>) compatible with the HLS design. Output: weights.h containing int16_t arrays W1, b1, W2, b2, W3, b3.

NOTE: The Q6.10 format has scale=1024, range -32.0 to +31.999. If saturation warnings appear, weights are clipped at the boundary.

## Step 1.4 — Generate Test Images

python image.py

Generates test_d_1.txt through test_d_50.txt (target letter images) and test_notd_1.txt through test_notd_50.txt (non-target images). These are used for HLS testbench and UART testing.

## Step 1.5 — Verify Fixed-Point Accuracy

python verify_fixed.py

Reads weights.h and runs fixed-point inference matching exactly what nn.cpp does on hardware. Should show accuracy above 90%. This confirms weights.h is correct before HLS synthesis.

STAGE 2: Vitis HLS

# Stage 2 — Vitis HLS Synthesis

## Step 2.1 — Copy weights.h to HLS Project

Copy the newly generated weights.h into your HLS source folder (same location as nn.cpp):

nn_classifier\  <- paste weights.h here alongside nn.cpp

The weights.h contains the Q6.10 fixed-point arrays that the HLS IP uses as constants.

## Step 2.2 — Key HLS Files

| File | Purpose | Change for new letter? |
| --- | --- | --- |
| nn.h | Data types: ap_fixed<16,6>, ap_uint<8> | No |
| nn.cpp | Network inference, HLS pragmas, AXI interfaces | No |
| weights.h | W1,b1,W2,b2,W3,b3 in Q6.10 format | YES - regenerate |
| nn_tb.cpp | C simulation testbench using test_d_*.txt files | No |

## Step 2.3 — nn.cpp AXI Interface Pragmas

The HLS top function uses these interface pragmas which determine how it connects in Vivado:

#pragma HLS INTERFACE m_axi     port=input  bundle=MAXI  depth=784

#pragma HLS INTERFACE s_axilite port=input  bundle=CTRL

#pragma HLS INTERFACE s_axilite port=result bundle=CTRL

#pragma HLS INTERFACE s_axilite port=return bundle=CTRL

- m_axi (MAXI): The IP fetches pixel data directly from DDR memory

- s_axilite (CTRL): The ARM writes the pixel buffer address and reads the result

## Step 2.4 — Run C Simulation

In Vitis HLS: Project -> Run C Simulation. This runs nn_tb.cpp against your C model. Should show 100 PASS results (50 target letter + 50 non-target).

## Step 2.5 — Run C Synthesis

In Vitis HLS: Solution -> Run C Synthesis. This synthesizes the C code into RTL (Verilog/VHDL). Check the synthesis report for:

- Timing: should meet the 100 MHz target (10 ns period)

- Utilization: approximately 1000-1100 LUTs, 3 DSPs, 24 BRAMs

- No critical warnings

## Step 2.6 — Export RTL

Solution -> Export RTL -> select Vivado IP format. This creates the IP zip file at:

nn_classifier\solution1\impl\ip\

The exported IP contains the RTL and the software driver files needed by Vitis.

## Step 2.7 — Fix the Driver Makefile (Windows only)

This is a known Vitis HLS 2020.1 bug on Windows. The generated Makefile uses shell globs that Windows CMD cannot expand. Open:

nn_classifier\solution1\impl\ip\drivers\nn_top_v1_0\src\Makefile

Delete all content and replace with (use TAB not spaces for indented lines):

COMPILER=

ARCHIVER=

CP=cp

COMPILER_FLAGS=

EXTRA_COMPILER_FLAGS=

LIB=libxil.a

RELEASEDIR=../../../lib

INCLUDEDIR=../../../include

INCLUDES=-I./. -I${INCLUDEDIR}

INCLUDEFILES=*.h

LIBSOURCES=$(wildcard *.c)

OBJECTS=$(addsuffix .o, $(basename $(wildcard *.c)))

libs:

echo "Compiling nn_top..."

$(COMPILER) $(COMPILER_FLAGS) $(EXTRA_COMPILER_FLAGS) $(INCLUDES) $(LIBSOURCES)

$(ARCHIVER) -r ${RELEASEDIR}/${LIB} ${OBJECTS}

make clean

include:

${CP} $(INCLUDEFILES) $(INCLUDEDIR)

clean:

rm -rf ${OBJECTS}

NOTE: The TAB character before echo, $(COMPILER), $(ARCHIVER), make clean, $(CP), rm -rf is critical. Spaces will cause Make to fail.

STAGE 3: Vivado Block Design

# Stage 3 — Vivado Block Design

## Step 3.1 — Open Existing Project (Recommended)

If you have an existing working project, open it and just refresh the HLS IP. This avoids all connection automation issues:

- Open nn_vivado.xpr in Vivado

- Tools -> Settings -> IP -> Repository -> click Refresh -> OK

- Open block design -> Reports -> Report IP Status

- Check nn_top_0 -> click Upgrade Selected -> OK

- Click Generate Bitstream

Skip to Step 3.9 if using existing project.

## Step 3.2 — Create New Project (If Needed)

- File -> Create Project -> Next

- Project name: nn_vivado

- Project type: RTL Project -> Next -> Next (skip adding sources)

- Board tab -> search Arty Z7-20 -> select -> Next -> Finish

## Step 3.3 — Add HLS IP Repository

Tools -> Settings -> IP -> Repository -> click + -> browse to:

nn_classifier\solution1\impl\ip\

Click Select -> OK. Vivado scans and finds nn_top IP.

## Step 3.4 — Create Block Design

IP Integrator -> Create Block Design -> Name: nn_design -> OK

## Step 3.5 — Add All 5 IP Blocks

Click + to add each block:

### Block 1: ZYNQ7 Processing System

- Search zynq -> double-click ZYNQ7 Processing System

- Click Run Block Automation green banner -> check Apply Board Preset -> OK

- Double-click Zynq block to verify settings:

  - PS-PL Configuration -> M AXI GP0: ENABLED

  - PS-PL Configuration -> S AXI HP0: ENABLED (required for m_axi)

  - Peripheral I/O Pins -> UART 1: ENABLED (USB-UART)

  - Clock Configuration -> FCLK_CLK0: 100 MHz

- Click OK

### Block 2: AXI Interconnect

- Search axi interconnect -> double-click

- Number of Slave Interfaces: 1

- Number of Master Interfaces: 2 (one for nn_top, one for AXI GPIO)

- Click OK

### Block 3: nn_top (Your HLS IP)

- Search nn_top -> double-click

- No configuration needed — ports are fixed by HLS code

- Exposes: s_axi_CTRL (control) and m_axi_MAXI (data fetch from DDR)

### Block 4: AXI GPIO

- Search axi gpio -> double-click

- IMPORTANT: Click Board tab first -> set BOTH GPIO and GPIO2 dropdowns to Custom

- Click IP Configuration tab and set:

  - GPIO (Channel 1 for LEDs): All Outputs checked, GPIO Width: 4

  - Default Output Value: 0x00000000, Default Tri State Value: 0x00000000

  - Check Enable Dual Channel

  - GPIO 2 (Channel 2 for Buttons): All Inputs checked, GPIO Width: 4

- Click OK

- Right-click GPIO port -> Make External -> rename to led

- Right-click GPIO2 port -> Make External -> rename to btn

### Block 5: Processor System Reset

- Search proc sys reset -> double-click

- No configuration needed

- Syncs PS and PL resets to prevent startup glitches

## Step 3.6 — Run Connection Automation

Click Run Connection Automation green banner -> check All Automation -> OK.

Verify these connections exist after automation:

| From | To |
| --- | --- |
| PS M_AXI_GP0 | AXI Interconnect S00_AXI |
| AXI Interconnect M00_AXI | nn_top s_axi_CTRL |
| AXI Interconnect M01_AXI | AXI GPIO S_AXI |
| nn_top m_axi_MAXI | PS S_AXI_HP0 (via axi_mem_intercon) |
| All clocks | FCLK_CLK0 (100 MHz) |
| All resets | rst_ps7_0_100M/peripheral_aresetn |

## Step 3.7 — Fix Address Editor

Click Address Editor tab at top. Both peripherals must have addresses assigned:

| Peripheral | Address | Range |
| --- | --- | --- |
| nn_top_0 (s_axi_CTRL) | 0x44A00000 | 64K |
| axi_gpio_0 | 0x40000000 or 0x41200000 | 64K |
| nn_top_0 m_axi_MAXI | 0x00000000 (DDR via HP0) | 512M |

If any show Unassigned, right-click -> Assign Address.

## Step 3.8 — Add XDC Constraints

File -> Add Sources -> Add or Create Constraints -> Create File -> name pins.xdc

Paste exactly (these are the Arty Z7-20 LED and button FPGA pin assignments):

set_property -dict { PACKAGE_PIN R14 IOSTANDARD LVCMOS33 } [get_ports { led_tri_o[0] }];

set_property -dict { PACKAGE_PIN P14 IOSTANDARD LVCMOS33 } [get_ports { led_tri_o[1] }];

set_property -dict { PACKAGE_PIN N16 IOSTANDARD LVCMOS33 } [get_ports { led_tri_o[2] }];

set_property -dict { PACKAGE_PIN M14 IOSTANDARD LVCMOS33 } [get_ports { led_tri_o[3] }];

set_property -dict { PACKAGE_PIN D19 IOSTANDARD LVCMOS33 } [get_ports { btn_tri_i[0] }];

set_property -dict { PACKAGE_PIN D20 IOSTANDARD LVCMOS33 } [get_ports { btn_tri_i[1] }];

set_property -dict { PACKAGE_PIN L20 IOSTANDARD LVCMOS33 } [get_ports { btn_tri_i[2] }];

set_property -dict { PACKAGE_PIN L19 IOSTANDARD LVCMOS33 } [get_ports { btn_tri_i[3] }];

NOTE: Port names must be led_tri_o and btn_tri_i exactly. AXI GPIO appends these suffixes automatically.

## Step 3.9 — Validate, Wrap, Generate Bitstream

- Click Validate Design (tick icon) -> must say Validation successful

- Sources panel -> right-click nn_design.bd -> Create HDL Wrapper -> Let Vivado manage -> OK

- Flow Navigator -> Generate Bitstream -> OK on all dialogs

- Wait 15-30 minutes for synthesis + implementation + bitstream

- When complete: File -> Export -> Export Hardware -> Fixed -> Include Bitstream -> Finish

Output file: nn_design_wrapper.xsa in your Vivado project folder.

NOTE: LUTAR-1 warnings (3 count) about asynchronous preset/clear are safe to ignore. RTSTAT-10 warnings about unroutable loads are also safe.

STAGE 4: Vitis IDE

# Stage 4 — Vitis IDE Setup

## Step 4.1 — Create New Workspace

Close any existing Vitis workspace. Create a new empty folder:

Desktop\projects\mnist\review_hls\nn_vitis_y\

Open Vitis 2020.1 -> Browse to the new folder -> Launch.

## Step 4.2 — Copy Driver Files Before Creating Platform

Navigate to the new workspace BSP folder (it may not exist yet — create it if needed):

nn_vitis_y\nn_platform\ps7_cortexa9_0\standalone_domain\bsp\ps7_cortexa9_0\libsrc\nn_top_v1_0\src\

Copy all 6 files from the HLS driver folder:

nn_classifier\solution1\impl\ip\drivers\nn_top_v1_0\src\

Files to copy: Makefile (the fixed one), xnn_top.c, xnn_top.h, xnn_top_hw.h, xnn_top_linux.c, xnn_top_sinit.c

Also copy xnn_top.h and xnn_top_hw.h into the include folder:

nn_vitis_y\nn_platform\ps7_cortexa9_0\standalone_domain\bsp\ps7_cortexa9_0\include\

## Step 4.3 — Create Platform Project

File -> New -> Platform Project:

| Setting | Value |
| --- | --- |
| Platform project name | nn_platform |
| Source | Create from hardware specification (XSA) |
| XSA file | nn_vivado\nn_design_wrapper.xsa |
| Operating system | standalone |
| Processor | ps7_cortexa9_0 |
| Generate boot components | UNCHECKED (critical) |

Click Finish.

## Step 4.4 — Build Platform

Right-click nn_platform -> Build Project. Console must show:

"Compiling nn_top..."

'Finished building libraries'

INFO: Build Successful

NOTE: If you see arm-none-eabi-ar: *.o: Invalid argument, the Makefile fix was not applied correctly. Check that indented lines use TAB characters, not spaces.

## Step 4.5 — Create Application Project

File -> New -> Application Project:

| Setting | Value |
| --- | --- |
| Platform | nn_platform |
| Application name | letter_y_app |
| Domain | standalone_ps7_cortexa9_0 |
| Template | Empty Application |

Click Finish.

## Step 4.6 — Create main.c

Expand letter_y_app -> right-click src -> New -> File -> name main.c -> Finish.

Paste the complete main.c code (see Appendix A for full code listing).

## Step 4.7 — Build Application

Right-click letter_y_app -> Build Project. Must end with:

Finished building: letter_y_app.elf

Expected ELF size: approximately 28000 bytes text, 1200 bytes data.

## Step 4.8 — Program FPGA

Connect Arty Z7-20 via USB. Then in Vitis:

Xilinx -> Program FPGA -> browse to bitstream:

nn_vivado\nn_vivado.runs\impl_1\nn_design_wrapper.bit

Click Program. The DONE LED on the board goes green.

## Step 4.9 — Run Application

Right-click letter_y_app -> Run As -> Launch on Hardware (Single Application Debug).

In XSCT Console at bottom right, type:

con

Watch the board: all 4 LEDs (LD0-LD3) blink together once then go off. This confirms the ARM is running and the Y detector is ready.

NOTE: If the application stops at a hardware breakpoint on main() line 26, pressing F8 or typing con in XSCT Console resumes execution.

STAGE 5: Python Testing

# Stage 5 — Python Testing and Canvas UI

## Step 5.1 — Test with Known Images

Run from the folder containing test_d_1.txt files:

python test_leds_y.py

Expected output:

TEST 1: Sending known Y image

Result: Y DETECTED   <- LD0 lights on board

TEST 2: Sending known NOT-Y image

Result: NOT Y        <- LD1 lights on board

The auto loop then alternates LD0 and LD1 every 3 seconds.

## Step 5.2 — COM Port Selection

The Arty Z7-20 creates two COM ports over the single USB cable (FT2232 chip):

| COM Port | Function |
| --- | --- |
| Lower number (e.g. COM16) | JTAG (used by Vivado/Vitis for programming) |
| Higher number (e.g. COM17) | UART (used by Python for pixel data) |

Always use the higher numbered COM port in canvas_ui.py and test scripts.

## Step 5.3 — Canvas UI

python canvas_ui.py

Select COM17 in the dropdown. Draw a letter in the 28x28 grid. Click Send to FPGA.

- Y DETECTED appears in green = LD0 lights on board

- NOT Y appears in red = LD1 lights on board

NOTE: The canvas applies np.fliplr() before sending to compensate for the torch.rot90(k=-1) applied during training. Draw a normal upright letter.

## Step 5.4 — Show Reference Image

To see exactly what shape the network expects:

python show_y.py

This loads test_d_1.txt (a confirmed Y image) onto the canvas with horizontal flip applied so it displays correctly. Click Send to confirm it detects, then Clear and try drawing the same shape.

## Step 5.5 — Full Accuracy Test

python test_real_images.py

Tests all 10 known Y images and 10 known not-Y images. Expected result: 20/20 = 100%.

STAGE 6: Changing Target Letter

# Stage 6 — Changing to a Different Letter

To change from detecting one letter to another (e.g. D to Y to F), follow this sequence:

## Files to Change

| File | Line to Change | Change Required? |
| --- | --- | --- |
| train.py | D_LABEL = X | YES |
| image.py | D_LABEL = X | YES |
| verify_fixed.py | D_LABEL = X | YES |
| export_weights.py | Nothing | No |
| nn.cpp | Nothing | No |
| nn.h | Nothing | No |
| nn_tb.cpp | Nothing | No |
| weights.h | Regenerated automatically | Auto |
| canvas_ui.py | Title text only | Optional |
| main.c | Nothing functional | No |

## Complete Change Sequence

- Change D_LABEL in train.py, image.py, verify_fixed.py

- python train.py -> generates new model.pth

- python export_weights.py -> generates new weights.h

- python image.py -> generates new test images

- python verify_fixed.py -> verify accuracy > 90%

- Copy new weights.h to HLS source folder

- Vitis HLS: C Simulation -> C Synthesis -> Export RTL

- Vivado: Tools -> Settings -> IP -> Refresh -> Upgrade nn_top_0 in block design

- Vivado: Generate Bitstream -> Export Hardware -> new XSA

- Vitis: Update Hardware Specification -> Build Platform -> Build App

- Program FPGA -> Run -> test with Python scripts

NOTE: Steps 1-5 take about 15 minutes total. Steps 6-8 (HLS + Vivado) take 30-60 minutes due to synthesis time.

TROUBLESHOOTING

# Common Issues and Fixes

| Error / Symptom | Fix |
| --- | --- |
| arm-none-eabi-ar: *.o: Invalid argument | Fix Makefile in HLS driver src folder — replace *.o with $(OBJECTS) using $(wildcard). See Stage 2 Step 2.7. |
| xpfm path is not valid | Build the platform project first before building the application. Right-click nn_platform -> Build Project. |
| ELF does not exist error | Build letter_y_app directly, not letter_y_app_system. Right-click the app project, not the system project. |
| UART timeout — no response from board | Type con in XSCT Console. ARM may be paused at breakpoint. Also verify correct COM port (higher number = UART). |
| LEDs not responding | LEDs are on PL fabric, not MIO pins. Requires AXI GPIO block in Vivado block design. Cannot use XGpioPs for these LEDs. |
| DRC NSTD-1 / UCIO-1 errors | XDC port names must match exactly: led_tri_o[0..3] and btn_tri_i[0..3]. Check for typos. |
| Board interface conflict in AXI GPIO | Set GPIO and GPIO2 Board dropdowns to Custom in IP Configuration tab before clicking OK. |
| Canvas always shows NOT Y/D | The network was trained on rotated images. The canvas applies fliplr() before sending. Draw a normal upright letter. |
| Both LEDs blink but no result from Python | The blink is the ARM startup signal. Python timed out waiting. Check COM port number and that ARM is running (con in XSCT). |
| Synthesis takes very long | Normal — 15-25 minutes for nn_top due to 50,000+ weight constants. Use launch_runs with -jobs 8 to parallelize. |

APPENDIX A: main.c Complete Code

# Appendix A — main.c

Complete bare-metal C code for the ARM Cortex-A9. Works for any target letter — no changes needed when switching letters.

#include "xparameters.h"

#include "xuartps.h"

#include "xnn_top.h"

#include "xil_cache.h"

#include "xgpio.h"

#include <string.h>

#define UART_DEVICE_ID   XPAR_XUARTPS_0_DEVICE_ID

#define NN_DEVICE_ID     XPAR_NN_TOP_0_DEVICE_ID

#define GPIO_DEVICE_ID   XPAR_AXI_GPIO_0_DEVICE_ID

#define PIX_ADDR     0x10000000

#define N_PIX        784

#define LED_CHANNEL  1

#define BTN_CHANNEL  2

static XUartPs  Uart;

static XNn_top  Nn;

static XGpio    Gpio;

int main(void)

{

u8 buf[N_PIX];

u8 response[1];

/* UART init */

XUartPs_Config *uc = XUartPs_LookupConfig(UART_DEVICE_ID);

XUartPs_CfgInitialize(&Uart, uc, uc->BaseAddress);

XUartPs_SetBaudRate(&Uart, 115200);

/* AXI GPIO init */

XGpio_Initialize(&Gpio, GPIO_DEVICE_ID);

XGpio_SetDataDirection(&Gpio, LED_CHANNEL, 0x0);

XGpio_SetDataDirection(&Gpio, BTN_CHANNEL, 0xF);

XGpio_DiscreteWrite(&Gpio, LED_CHANNEL, 0x0);

/* HLS IP init */

XNn_top_Config *nc = XNn_top_LookupConfig(NN_DEVICE_ID);

XNn_top_CfgInitialize(&Nn, nc);

/* Startup blink — all 4 LEDs = ready signal */

XGpio_DiscreteWrite(&Gpio, LED_CHANNEL, 0xF);

for (volatile int i = 0; i < 50000000; i++);

XGpio_DiscreteWrite(&Gpio, LED_CHANNEL, 0x0);

while (1)

{

/* 1. Receive 784 bytes from Python */

u32 got = 0;

while (got < N_PIX)

got += XUartPs_Recv(&Uart, buf + got, N_PIX - got);

/* 2. Copy to DDR and flush cache */

memcpy((void *)PIX_ADDR, buf, N_PIX);

Xil_DCacheFlushRange((UINTPTR)PIX_ADDR, N_PIX);

/* 3. Run neural network */

XNn_top_Set_input_r(&Nn, (u64)PIX_ADDR);

XNn_top_Start(&Nn);

while (!XNn_top_IsDone(&Nn));

/* 4. Drive LEDs: LD0=detected, LD1=not detected */

u32 r = XNn_top_Get_result_o(&Nn);

if (r == 1)

XGpio_DiscreteWrite(&Gpio, LED_CHANNEL, 0x1);

else

XGpio_DiscreteWrite(&Gpio, LED_CHANNEL, 0x2);

/* 5. Send result back to Python */

response[0] = (u8)r;

XUartPs_Send(&Uart, response, 1);

}

return 0;

}

APPENDIX B: Block Design Summary

# Appendix B — Complete Block Design Settings

| Block | Key Settings | Purpose |
| --- | --- | --- |
| ZYNQ7 PS | Board preset, UART1 on, S_AXI_HP0 on, M_AXI_GP0 on, FCLK_CLK0=100MHz | ARM processor, DDR, clocks |
| AXI Interconnect | 1 slave, 2 masters | Routes AXI bus between PS and peripherals |
| nn_top HLS IP | No GUI settings — ports fixed by HLS | Neural network inference in FPGA fabric |
| AXI GPIO | CH1: 4-bit output (LEDs), CH2: 4-bit input (buttons), Board=Custom | Drives LD0-LD3 and reads BTN0-BTN3 |
| Proc Sys Reset | No settings | Synchronizes PS and PL reset sequences |

## Address Map

| Peripheral | Base Address | Size |
| --- | --- | --- |
| nn_top_0 (s_axi_CTRL) | 0x44A00000 | 64K |
| axi_gpio_0 | 0x40000000 | 64K |
| DDR (pixel buffer) | 0x10000000 (software) | 784 bytes |

## XDC Pin Assignments

| Signal | Port Name | FPGA Pin | Standard |
| --- | --- | --- | --- |
| LD0 | led_tri_o[0] | R14 | LVCMOS33 |
| LD1 | led_tri_o[1] | P14 | LVCMOS33 |
| LD2 | led_tri_o[2] | N16 | LVCMOS33 |
| LD3 | led_tri_o[3] | M14 | LVCMOS33 |
| BTN0 | btn_tri_i[0] | D19 | LVCMOS33 |
| BTN1 | btn_tri_i[1] | D20 | LVCMOS33 |